# HomeSOClab — Network Architecture (Phase 1, Layer 2)

This document describes the segmented network design for the HomeSOClab. The design follows a zero-trust-inspired segmentation model: every zone is its own VLAN/L2 domain, and the firewall is the single inter-VLAN router enforcing an explicit allow-list policy (default deny).

## 1. Design Goals & Security Rationale

| Goal | How it is achieved |
| --- | --- |
| Contain malware/exploits to the lab | Lab VLANs cannot reach the home LAN (192.168.1.0/24) |
| Realistic attack path | Attacker VLAN → Victim VLAN allowed; Victim cannot initiate back |
| Prevent victim exfiltration / botnet C2 | Victim VLAN has no internet (default-deny WAN) |
| Full packet visibility for detection | Port mirroring (SPAN) copies inter-VLAN traffic to the SIEM sensor |
| Safe administration | Management VLAN is the only plane allowed to reach Proxmox/pfSense/SIEM admin UIs |
| Blast-radius reduction | Default-deny between every pair of zones; only documented flows are permitted |

## 2. VLAN / IP Addressing Scheme

| VLAN ID | Zone | Subnet (CIDR) | Gateway (pfSense) | DHCP Pool | Internet | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | 10.10.10.1 | .100–.199 | Controlled | Proxmox, pfSense, SIEM UI, analyst WS |
| 20 | Attacker | 10.10.20.0/24 | 10.10.20.1 | .100–.199 | ✅ Yes | Kali / red-team tooling |
| 30 | Victim | 10.10.30.0/24 | 10.10.30.1 | .100–.199 | ❌ No | Vulnerable endpoints / targets |
| 99 | Monitoring | 10.10.99.0/24 | 10.10.99.1 | static only | ❌ No | SPAN sink + SIEM sensor sniff NIC |

### Reserved static addresses (outside DHCP pools)

| Host | IP | VLAN |
| --- | --- | --- |
| pfSense/OPNsense (router) | 10.10.X.1 per VLAN | all |
| Proxmox pve01 mgmt | 10.10.10.2 | 10 |
| SIEM management UI | 10.10.10.10 | 10 |
| Analyst workstation | 10.10.10.20 | 10 |
| SIEM sensor (mirror sink NIC) | 10.10.99.10 | 99 |

Why a separate Monitoring VLAN (99)? The SIEM sensor’s sniffing interface receives mirrored traffic and must never source traffic onto production zones. Its management interface lives on VLAN 10 so analysts can reach the dashboard. This dual-NIC pattern mirrors how Security Onion / Wazuh are deployed in production (a management NIC + a promiscuous monitor NIC).

## 3. Topology Diagram

INTERNET
                                 │
                       ┌─────────┴─────────┐
                       │   Home Router      │   192.168.1.0/24  (UNTRUSTED to lab)
                       │  (existing LAN)    │
                       └─────────┬─────────┘
                                 │ WAN (DHCP)
                                 ▼
            ╔════════════════════════════════════════════════╗
            ║         pfSense / OPNsense  (Firewall)          ║
            ║   - Inter-VLAN router (router-on-a-stick)       ║
            ║   - Default-deny + explicit allow rules         ║
            ║   - DHCP + Unbound DNS per VLAN                 ║
            ║   - NAT (outbound) for MGMT & ATTACKER only     ║
            ╚══╤═════════╤═══════════╤════════════╤═══════════╝
       VLAN10 │  VLAN20 │   VLAN30  │   VLAN99   │   (802.1Q trunk)
        MGMT  │ ATTACKER│   VICTIM  │ MONITORING │
              │         │           │            │
              ▼         ▼           ▼            ▼
        ┌───────────────────────────────────────────────────┐
        │      Proxmox VE host  —  vmbr1 (VLAN-aware)         │
        │   physical trunk NIC enp1s0  ◄──► pfSense vNIC      │
        └──┬──────────┬───────────┬──────────────┬───────────┘
           │          │           │              │
   ┌───────▼──┐  ┌────▼─────┐ ┌───▼──────┐  ┌────▼───────────┐
   │ Proxmox  │  │  Kali    │ │ Victim   │  │ SIEM Sensor    │
   │ mgmt UI  │  │ Attacker │ │ Endpoints│  │ (mon NIC=vlan99│
   │ SIEM UI  │  │ 10.10.20 │ │ 10.10.30 │  │  mgmt NIC=v10) │
   │ Analyst  │  │   .x     │ │   .x     │  │ 10.10.99.10    │
   │ 10.10.10 │  └──────────┘ └──────────┘  └────────────────┘
   │   .x     │       ▲             ▲              ▲
   └──────────┘       │             │              │
                      └── attack ──►│              │
                                    │              │
        Mirrored copies of inter-VLAN traffic ─────┘
        (pfSense pflog/SPAN OR Proxmox bridge mirror → VLAN99)

## 4. Traffic Flow Matrix (Firewall Policy Summary)

✅ = allowed, ❌ = blocked (default deny), ⚠️ = restricted to specific ports

| From ↓ To → | MGMT | ATTACKER | VICTIM | MONITORING | Home LAN | Internet |
| --- | --- | --- | --- | --- | --- | --- |
| MGMT | — | ✅ | ✅ | ⚠️ | ❌ | ✅ |
| ATTACKER | ❌ | — | ✅ | ❌ | ❌ | ✅ |
| VICTIM | ❌ | ❌ | — | ❌ | ❌ | ❌ |
| MONITORING | ❌ | ❌ | ❌ | — | ❌ | ❌ |

Key policy decisions: - Management → everything: admins/SIEM must reach all zones for triage. - Attacker → Victim only: realistic kill-chain; attacker cannot touch the admin plane or the home LAN (prevents the lab from becoming a pivot point). - Victim → nothing outbound: blocks C2/exfil; only responds to attacker. - Monitoring isolated: sensor only receives mirrored frames; it never routes. Its mgmt UI is reached from VLAN10, not from VLAN99.

See pfsense/rules/ for the concrete rule definitions per interface.

## 5. Traffic Mirroring (Network Telemetry Feed)

Phase 1 supports two complementary mirroring approaches (see pfsense/port-mirror.md and proxmox/bridge-mirror.md):

- Proxmox bridge mirroring (recommended, hypervisor-level): a tc-based SPAN that copies all frames traversing vmbr1 to a dedicated mirror port connected to the SIEM sensor’s monitor NIC (VLAN 99). Captures all VM traffic including intra-VLAN east-west traffic.

- pfSense span/packet capture (firewall-level): mirrors traffic crossing the firewall to a monitor interface. Simpler but only sees inter-VLAN/WAN traffic, not intra-VLAN.

Both deliver line-rate copies to 10.10.99.10 where the SIEM sensor runs in promiscuous mode (Zeek/Suricata) in later phases.